IT Service Ticket – Memory (RAM) Replacement & Validation

Ticket ID: LAB-HW-RAM-001
Category: Hardware → Memory (RAM)
Priority: Medium
Status: Completed – Verified
Reported By: Phillip Bridgeman
Assigned Technician: Phillip Bridgeman
Date Opened: 2025-12-21
Date Closed: 2025-12-21

Summary

Replaced aging DDR3 memory modules with higher-capacity RAM on a legacy AMD-based system. Performed BIOS-level validation to confirm proper detection, speed, slot population, and dual-channel operation. Upgrade completed successfully with no POST or stability issues observed.

Environment Details

- System Type: Legacy desktop (home lab)

- Motherboard: ASRock N68C-S UCC

- BIOS Version: P1.40 (Legacy BIOS)

- Processor: AMD Athlon™ II X2 255 (64-bit, 3.1 GHz)

- Firmware Mode: Legacy BIOS

- Integrated Graphics: Enabled (256 MB shared memory)

Original RAM Configuration (Removed)

- Manufacturer: Kingston

- Model: Kingston ValueRAM (KVR1333D3N9/2G)

- Type: DDR3 SDRAM

- Capacity: 2 × 2 GB (4 GB total)

- Speed: DDR3-1333 (PC3-10600)

- Voltage: 1.5 V

- Form Factor: 240-pin DIMM

- ECC: Non-ECC, Unbuffered

- Condition: One module exhibited visible surface contamination consistent with age and environmental exposure

New RAM Installed

- Manufacturer: Timetec

- Type: DDR3 SDRAM

- Capacity: 2 × 4 GB (8 GB total)

- Rated Speed: DDR3-1600 (operating at DDR3-1333 due to platform limitations)

- Voltage: 1.35 V (DDR3L, backward compatible)

- Form Factor: 240-pin DIMM

- ECC: Non-ECC, Unbuffered

Scope of Work

- Remove existing RAM modules

- Install new memory modules

- Verify slot placement for dual-channel operation

- Validate memory detection and speed via BIOS

- Confirm system stability on boot

Actions Taken

- Powered down system and disconnected AC power.

- Discharged residual power and followed ESD-safe handling procedures.

- Removed existing DDR3 memory modules.

- Installed new 4 GB DDR3 DIMMs into matching channel slots (A1/B1).

- Powered on system and entered BIOS setup utility.

- Verified total memory capacity and per-slot detection.

- Confirmed dual-channel memory mode operation.

- Booted system successfully with no POST errors.

Validation & Testing Results

- Total Memory Detected: 8192 MB (8 GB)

- Slot Detection:

  - DDR3_A1: 4096 MB @ 667 MHz (DDR3-1333)

  - DDR3_B1: 4096 MB @ 667 MHz (DDR3-1333)

- Dual-Channel Mode: Enabled

- POST Errors: None

- System Boot: Successful

- Stability (Initial Runtime): Stable

Note: Reported 667 MHz clock rate reflects DDR architecture (667 MHz × 2 = DDR3-1333), which is expected behavior.

Results

Memory upgrade completed successfully. System correctly recognizes and utilizes the full 8 GB of installed RAM in dual-channel mode. No errors or compatibility issues detected. System performance and capacity improved relative to original configuration.

Risks / Notes

- Platform chipset limits memory frequency to DDR3-1333.

- DDR3L modules correctly downshifted voltage and speed for compatibility.

- Integrated graphics reserves 256 MB shared system memory (expected behavior).

Recommendations / Next Steps

- Optional: Run extended memory diagnostics (e.g., MemTest86) for long-duration stability validation.

- Document final memory configuration in asset inventory.

- Proceed with OS-level testing or additional hardware upgrades as planned.